Actual Weights

JHSV-B043b

Rev3

- Introduction

The Seller Data Requirements List (SDRL) identifies Seller Data Item requirements in support of the Joint High Speed Vessel (JHSV) program.  Emergent issues that are not addressed herein or otherwise provided for in source documents shall be referred to Austal USA Buyer. Data shall be provided under this Purchase Order/Subcontract.

NOTICE:

All Sellers who have interest in utilizing foreign resources for the purposes of supplying equipment, materials, and/or services are hereby notified that information contained within the Purchase Order/Subcontract, to include all attachments, amendments, or any other information that is posted with a Purchase Order/Subcontract is subject to the International Traffic in Arms Regulation. Defense-related information of any kind may not be exported without the explicit permission via export license from the United States State Department.  For additional information about ITAR and specific requirements,

visit http://www.pmddtc.state.gov/itar_index.htm

| 1. | SDRL Title: | Actual Weights |
| --- | --- | --- |
| 2. | SDRL Number: | B043b |
| 3. | First Submittal: | At delivery of equipment |
| 4. | Subsequent Submittals: | N/A. |
| 5. | Comment Responses | All comments shall be responded to 5 days after comments received. Revisions shall incorporate resolution of comments |
| 6. | Distribution: | 1 reproducible electronic copy |

7.	Remarks & Preparation Instructions

The Seller shall provide actual weights of all items it provides to the Buyer. The attached “Actual Weight Data Sheet” is the required format for providing weight reports. “Wet” and “Dry” weights for each item shall be provided as applicable.

- “Wet” weight is the weight of the item with all operating fluids (oil, fuel, water, etc.) at full design level.

- “Dry” weight is the weight of the item with all operating fluids completely removed.

After the item has been completely manufactured and assembled, and prior to shipment, the Seller shall determine the item’s actual weight by using either certified scales or a certified load cell. The item shall be weighed in both the “wet” and “dry” conditions (if applicable). These weights shall be recorded on the “Actual Weight Data Sheet”. The item shall be weighed without any packaging or protective material attached.

The Seller shall provide the inertia of the item about its own center of gravity for all items with an estimated unit weight greater than 1,000 kg. The Seller shall refer to SAWE document SAWE_RP13_5Jun1996.pdf for calculating the moments of inertia.

| NAME & PART NUMBER OF ITEM | QTY | “DRY” WEIGHT (KG) | “WET” WEIGHT (KG) | INERTIA |
| --- | --- | --- | --- | --- |